🔬 Cell Counter Using Watershed Algorithm

Project Explanation & Code Walkthrough

# 1. Project Overview

This project implements an automated cell counting system using classical computer vision techniques in Python and OpenCV. The primary goal is to accurately count individual cells in a microscopy image, even when cells are closely packed or partially overlapping.

The core algorithm is the Watershed Segmentation Algorithm — a well-known technique in image processing that treats pixel intensities like a topographic map and finds boundaries between objects (cells) by simulating flooding from known seed points.

# 2. Technologies Used

- Python 3.7+ — Programming language

- OpenCV (cv2) — Image processing and computer vision library

- NumPy — Numerical array operations

# 3. Algorithm Pipeline — Step by Step

The cell counting pipeline follows 13 sequential steps. Each step transforms the image to make cells progressively easier to detect and count.

| Step | Stage | OpenCV Function | Purpose |
| --- | --- | --- | --- |
| 1 | Load Image | cv2.imread() | Load the input microscopy image from disk |
| 2 | Grayscale | cv2.cvtColor() | Convert BGR image to grayscale for processing |
| 3 | Blur / Denoise | cv2.GaussianBlur() | Smooth image to reduce random noise |
| 4 | Threshold | cv2.adaptiveThreshold() | Binarize image adaptively — cells become white |
| 5 | Morphology | cv2.morphologyEx(OPEN) | Remove small noise specks using erosion+dilation |
| 6 | Background | cv2.dilate() | Expand to guarantee the background region |
| 7 | Distance Map | cv2.distanceTransform() | Measure distance to nearest zero pixel per cell |
| 8 | Foreground | cv2.threshold() | Identify sure cell centers from distance map peaks |
| 9 | Unknown Zone | cv2.subtract() | Find ambiguous border region between cells |
| 10 | Markers | cv2.connectedComponents() | Label each foreground region with a unique ID |
| 11 | Watershed | cv2.watershed() | Flood-fill from markers to segment touching cells |
| 12 | Count | np.unique(markers) | Count unique segments minus borders and background |
| 13 | Draw Borders | image[markers == -1] | Overlay red lines on watershed boundaries |

# 4. Detailed Code Explanation

## Step 1: Load Image

image = cv2.imread("cells.jpg")

Loads the image from disk into a NumPy array in BGR (Blue-Green-Red) format. If the file is not found, the script exits with an error message.

## Step 2: Convert to Grayscale

gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

Most image analysis works on single-channel (grayscale) images. Converting removes color information and gives a 2D intensity array, making subsequent operations simpler and faster.

## Step 3: Gaussian Blur

blur = cv2.GaussianBlur(gray, (5, 5), 0)

Applies a 5×5 Gaussian filter to smooth out high-frequency noise (dust, sensor artifacts). This prevents small noise pixels from being mistaken for cell boundaries during thresholding.

## Step 4: Adaptive Thresholding

thresh = cv2.adaptiveThreshold(blur, 255, cv2.ADAPTIVE_THRESH_GAUSSIAN_C, cv2.THRESH_BINARY_INV, 11, 2)

Converts the grayscale image to a binary (black/white) image. Unlike global thresholding, adaptive thresholding calculates a different threshold for each small region of the image, which handles uneven lighting across the slide. Cells become white (255) on a black background.

## Step 5: Morphological Opening

kernel = np.ones((3,3), np.uint8)

opening = cv2.morphologyEx(thresh, cv2.MORPH_OPEN, kernel, iterations=2)

Morphological opening = erosion followed by dilation. This removes tiny white specks (noise) that are smaller than the 3×3 kernel while preserving the larger cell regions. Two iterations make the effect stronger.

## Step 6: Sure Background

sure_bg = cv2.dilate(opening, kernel, iterations=3)

Expands (dilates) the white regions outward by 3 iterations. This guarantees that every pixel in sure_bg is either a cell or a close neighbor of one — i.e., definitely NOT background far away from cells.

## Step 7: Distance Transform

dist_transform = cv2.distanceTransform(opening, cv2.DIST_L2, 5)

For every white (cell) pixel, computes its Euclidean distance to the nearest black (background) pixel. The result is a grayscale map where the brightest pixels are at the centers of cells — far from any edge.

## Step 8: Sure Foreground

_, sure_fg = cv2.threshold(dist_transform, 0.5*dist_transform.max(), 255, 0)

Thresholds the distance map: pixels with distance > 50% of the maximum are considered cell centers. These become the 'seed' markers for watershed. Using 0.5 as the factor balances sensitivity — lower values detect more seeds (risk over-splitting), higher values detect fewer (risk merging cells).

## Step 9: Unknown Region

unknown = cv2.subtract(sure_bg, sure_fg)

Subtracts the sure foreground from the sure background. The remaining pixels are in an ambiguous zone — they might be cell edges, inter-cell gaps, or touching boundaries. Watershed will decide these.

## Step 10: Marker Labelling

_, markers = cv2.connectedComponents(sure_fg)

markers = markers + 1

markers[unknown == 255] = 0

Labels each connected region in the foreground with a unique integer ID (1, 2, 3...). Adding 1 shifts IDs so background becomes 1 (not 0). Unknown pixels are set to 0, which tells watershed to classify them.

## Step 11: Apply Watershed

markers = cv2.watershed(image, markers)

The watershed algorithm floods from each labeled seed outward, competing between neighbors. Where two floods meet, a boundary is drawn. Boundary pixels are marked -1. This step separates touching cells even when they share a border.

## Step 12: Count Cells

cell_count = len(np.unique(markers)) - 2

Counts unique integer values in the markers array. We subtract 2 to exclude: label 1 (background) and label -1 (watershed boundary lines). The remaining unique values each represent one individual cell.

## Step 13: Draw Boundaries

image[markers == -1] = [0, 0, 255]

Paints all watershed boundary pixels red (BGR: 0, 0, 255) on the original color image. This visually confirms where the algorithm drew the cell borders, useful for validation and debugging.

# 5. Full Source Code

Below is the complete, annotated cell_counter.py file:

import cv2

import numpy as np

def count_cells(image_path):

# Step 1: Load image

image = cv2.imread(image_path)

if image is None:

print(f"Error: Image not found at '{image_path}'")

return None, None

# Step 2: Convert to grayscale

gray = cv2.cvtColor(image, cv2.COLOR_BGR2GRAY)

# Step 3: Noise removal

blur = cv2.GaussianBlur(gray, (5, 5), 0)

# Step 4: Adaptive threshold

thresh = cv2.adaptiveThreshold(

blur, 255,

cv2.ADAPTIVE_THRESH_GAUSSIAN_C,

cv2.THRESH_BINARY_INV, 11, 2

)

# Step 5: Remove noise using morphology

kernel = np.ones((3, 3), np.uint8)

opening = cv2.morphologyEx(thresh, cv2.MORPH_OPEN, kernel, iterations=2)

# Step 6: Sure background

sure_bg = cv2.dilate(opening, kernel, iterations=3)

# Step 7: Distance transform

dist_transform = cv2.distanceTransform(opening, cv2.DIST_L2, 5)

# Step 8: Sure foreground

_, sure_fg = cv2.threshold(dist_transform, 0.5*dist_transform.max(), 255, 0)

sure_fg = np.uint8(sure_fg)

# Step 9: Unknown region

unknown = cv2.subtract(sure_bg, sure_fg)

# Step 10: Marker labelling

_, markers = cv2.connectedComponents(sure_fg)

markers = markers + 1

markers[unknown == 255] = 0

# Step 11: Apply Watershed

markers = cv2.watershed(image, markers)

# Step 12: Count cells

cell_count = len(np.unique(markers)) - 2

# Step 13: Draw boundaries

result = image.copy()

result[markers == -1] = [0, 0, 255]

return cell_count, result

if __name__ == "__main__":

IMAGE_PATH = "cells.jpg"

count, result_image = count_cells(IMAGE_PATH)

if count is not None:

print(f"Accurate Cell Count: {count}")

cv2.imshow("Cell Detection Result", result_image)

cv2.waitKey(0)

cv2.destroyAllWindows()

# 6. How to Run the Project

## Install Dependencies

pip install opencv-python numpy

## Run the Script

python cell_counter.py

Make sure cells.jpg is in the same directory. The script will print the cell count and display the result image with red boundary lines overlaid on the detected cells.

# 7. Summary

This project demonstrates how classical image processing can accurately solve a real biological problem — counting cells — without requiring any deep learning or labelled training data. The Watershed algorithm is particularly suited for microscopy images because:

- It handles touching or overlapping cells well

- It works on standard grayscale microscopy images

- It requires no training data or pre-trained models

- It is fast and runs on any standard PC

The pipeline is also easily extensible — for example, cell area, shape, or intensity statistics could be extracted from each labeled marker region for further biological analysis.